scheng

Single-Board Computer Deployment Guide

Raspberry Pi 5 · NVIDIA Jetson Orin · Performance tuning for embedded hardware

scheng SDK v0.1.0

# 1. Why SBC Deployment Matters

Permanent video installations require hardware that is silent, compact, reliable, and inexpensive enough to leave running indefinitely. Single-board computers (SBCs) fit this profile better than desktop machines for many installation contexts — they run cool, consume 5–15W, cost under $200, and can be hidden inside a display enclosure or mounted behind a projector.

scheng targets two SBC platforms: the Raspberry Pi 5 (ARM Cortex-A76, VideoCore VII GPU) and the NVIDIA Jetson Orin series (ARM Cortex-A78, Ampere GPU). Both run Linux and support Vulkan — the graphics backend wgpu uses on Linux.

NOTE SBC GPU performance is substantially lower than Apple Silicon. The key is choosing the right render scale and shader complexity for the hardware. A well-tuned scheng instrument on a Pi 5 runs smoothly at 720p. On a Jetson Orin NX it reaches comfortable 1080p.

# 2. Hardware Reference

| Hardware | GPU | RAM | Price | Target resolution |
| --- | --- | --- | --- | --- |
| Raspberry Pi 5 | VideoCore VII (no Vulkan CTS) | 4–8 GB | $60–80 | 720p with 2D shaders |
| Raspberry Pi 5 + M.2 HAT | VideoCore VII | 8 GB | ~$110 | 720p, better sustained perf |
| NVIDIA Jetson Orin Nano | 1024-core Ampere | 4–8 GB | $250–500 | 1080p 2D, 720p raymarcher |
| NVIDIA Jetson Orin NX | 1024-core Ampere | 8–16 GB | $500–900 | 1080p raymarcher |
| NVIDIA Jetson AGX Orin | 2048-core Ampere | 32–64 GB | $999+ | 4K 2D, 1080p raymarcher |
| M1 Mac Mini (reference) | Apple M1 GPU | 8–16 GB | $700 | 1080p raymarcher native |

NOTE The Raspberry Pi 5 uses VideoCore VII which does NOT have a complete Vulkan Conformance Test Suite (CTS) pass as of early 2025. wgpu falls back to OpenGL ES via the GL backend. Set WGPU_BACKEND=gl before running.

# 3. Raspberry Pi 5

## 3.1 System Setup

Start with Raspberry Pi OS (64-bit, Bookworm). Install the Vulkan Mesa driver and Rust toolchain:

# Install Mesa Vulkan driver

sudo apt install -y mesa-vulkan-drivers vulkan-tools

# Verify Vulkan (expect: radv or v3dv driver listed)

vulkaninfo --summary 2>/dev/null | grep -i driver

# Install Rust

curl --proto '=https' --tlsv1.2 -sSf https://sh.rustup.rs | sh

source ~/.cargo/env

# Install ffmpeg (required for video/stream features)

sudo apt install -y ffmpeg libavcodec-dev libavformat-dev libavutil-dev

## 3.2 wgpu Backend

The Pi 5's VideoCore VII uses the v3dv Vulkan driver, which is functional but not Vulkan CTS certified. Set the backend explicitly:

export WGPU_BACKEND=gl

# or for Vulkan (less stable on Pi 5):

export WGPU_BACKEND=vulkan

TIP Add WGPU_BACKEND=gl to /etc/environment or your launch script so it's always set on boot.

## 3.3 Performance Settings

The Pi 5's GPU is capable for 2D shaders at 720p with appropriate tuning. Raymarching at native resolution is not viable — use render scale.

| Shader type | Recommended settings | Expected FPS at 720p |
| --- | --- | --- |
| 2D (plasma, noise, lines) | --render-scale 0.75 --msaa 1 | ~30–45 fps |
| 2D SDF (scheng-sdf) | --render-scale 0.75 --msaa 1 | ~25–40 fps |
| Raymarcher (scheng-feedback) | --render-scale 0.33 + 32 steps | ~20–30 fps |
| Raymarcher at 1080p | --render-scale 0.25 + 32 steps | ~15–20 fps |

## 3.4 Shader Optimization for Pi 5

Edit scene.frag to reduce march steps from 64 to 32 for raymarching templates:

// In scene.frag — reduce for Raspberry Pi 5

for(int i=0;i<32;i++){   // was 64, reduce to 32 for Pi 5

int samples = 1;          // disable motion blur on Pi 5

## 3.5 Installation Launch Script

#!/bin/bash

# /home/pi/launch-installation.sh

# scheng installation script for Raspberry Pi 5

export WGPU_BACKEND=gl

export RUST_LOG=warn  # reduce log noise in production

cd /home/pi/SPLASH/scheng-sdf

./target/release/scheng-sdf \

--width 1280 \

--height 720 \

--render-scale 0.75

# Auto-restart on crash (add to crontab or systemd service)

# @reboot /home/pi/launch-installation.sh

## 3.6 Systemd Service

For permanent installations, run scheng as a systemd service so it starts on boot and restarts on crash:

[Unit]

Description=scheng video installation

After=graphical.target

[Service]

User=pi

Environment=WGPU_BACKEND=gl

Environment=RUST_LOG=warn

WorkingDirectory=/home/pi/SPLASH/scheng-sdf

ExecStart=/home/pi/SPLASH/scheng-sdf/target/release/scheng-sdf --width 1280 --height 720 --render-scale 0.75

Restart=always

RestartSec=3

[Install]

WantedBy=graphical.target

# Install and enable

sudo cp scheng.service /etc/systemd/system/

sudo systemctl enable scheng

sudo systemctl start scheng

# 4. NVIDIA Jetson Orin

## 4.1 System Setup

Flash JetPack 6.x (Ubuntu 22.04-based) using NVIDIA SDK Manager or Balena Etcher with the official image. JetPack includes CUDA, Vulkan drivers, and display support out of the box.

# Verify Vulkan support

vulkaninfo --summary | grep -i driver

# Expected: NVIDIA Vulkan driver

# Install Rust

curl --proto '=https' --tlsv1.2 -sSf https://sh.rustup.rs | sh

source ~/.cargo/env

# Install ffmpeg

sudo apt install -y ffmpeg libavcodec-dev libavformat-dev libavutil-dev

## 4.2 wgpu Backend

Jetson Orin has full Vulkan support via NVIDIA drivers. wgpu selects Vulkan automatically — no environment variable needed:

# No WGPU_BACKEND override needed on Jetson

# wgpu auto-selects Vulkan with NVIDIA driver

## 4.3 Performance Settings

| Hardware | Shader type | Recommended settings | Expected FPS |
| --- | --- | --- | --- |
| Jetson Orin Nano | 2D shaders | --render-scale 1.0 at 1080p | ~45–60 fps |
| Jetson Orin Nano | Raymarcher | --render-scale 0.5 at 1080p | ~30–45 fps |
| Jetson Orin NX | 2D shaders | --render-scale 1.0 at 1080p | ~60+ fps |
| Jetson Orin NX | Raymarcher | --render-scale 0.75 at 1080p | ~40–60 fps |
| Jetson AGX Orin | 2D shaders | --render-scale 1.0 at 4K | ~30–45 fps |
| Jetson AGX Orin | Raymarcher | --render-scale 0.5 at 4K | ~30–40 fps |

TIP Jetson Orin supports HDMI 2.0 and DisplayPort. For 4K installations at 60Hz, use the AGX Orin or pair an Orin NX with --render-scale 0.5 and reduce march steps to 48.

## 4.4 Jetson Power Mode

Set the Jetson to maximum power mode for installations to ensure consistent GPU clock speeds:

# Set to MAXN (maximum performance) power mode

sudo nvpmodel -m 0

# Lock GPU and CPU clocks for consistent performance

sudo jetson_clocks

# Verify

sudo tegrastats

## 4.5 Installation Launch Script

#!/bin/bash

# /home/orin/launch-installation.sh

# Lock clocks for consistent frame timing

sudo jetson_clocks 2>/dev/null || true

export RUST_LOG=warn

cd /home/orin/SPLASH/scheng-feedback

./target/release/scheng-feedback \

--width 1920 \

--height 1080 \

--render-scale 0.75

# 5. Cross-Compilation

Building on the target hardware is the most reliable approach. However, for CI/CD or when the SBC is deployed and inaccessible, cross-compilation from a Mac or Linux host is possible.

## 5.1 Raspberry Pi 5 from macOS

# Install the cross-compilation toolchain

brew install arm-linux-gnueabihf-binutils

cargo install cross

# Build

cross build --release --target aarch64-unknown-linux-gnu

# Copy binary to Pi

scp target/aarch64-unknown-linux-gnu/release/scheng-sdf pi@raspberrypi.local:~/

NOTE Cross-compilation requires matching system libraries. The wgpu and ffmpeg dependencies have complex C library requirements. Building directly on the SBC is recommended for production.

# 6. Display Configuration

## 6.1 Headless / Framebuffer Output

For installations where the SBC has no attached display during setup, use a dummy HDMI plug to force display initialization:

- Plug a dummy HDMI adapter (available for ~$10) into the HDMI port

- The system will detect a display and initialize the GPU correctly

- scheng's preview window will render to the framebuffer

- Remove dummy plug and connect real display — window appears

## 6.2 Multi-Display

For multi-display installations with a single SBC, run multiple scheng instances, each targeting a different display:

# Display 0 (HDMI-1)

DISPLAY=:0.0 ./scheng-sdf --width 1920 --height 1080 &

# Display 1 (HDMI-2) — if available on hardware

DISPLAY=:0.1 ./scheng-sdf --width 1920 --height 1080 &

# 7. Quick Reference

## Pi 5 — Optimal Settings by Use Case

| Use case | Width | Height | --render-scale | shader steps | Expected FPS |
| --- | --- | --- | --- | --- | --- |
| Simple 2D loop | 1280 | 720 | 0.75 | N/A | 30–45 |
| SDF geometry | 1280 | 720 | 0.75 | N/A | 25–40 |
| Feedback/trails | 1280 | 720 | 0.5 | N/A | 25–35 |
| Raymarcher | 1280 | 720 | 0.33 | 32 | 20–30 |
| 4K display output | 3840 | 2160 | 0.2 | N/A | 15–25 |

## Jetson Orin NX — Optimal Settings by Use Case

| Use case | Width | Height | --render-scale | Expected FPS |
| --- | --- | --- | --- | --- |
| Simple 2D loop | 1920 | 1080 | 1.0 | 60+ |
| SDF geometry | 1920 | 1080 | 1.0 | 55–65 |
| Raymarcher | 1920 | 1080 | 0.75 | 40–55 |
| 4K display output | 3840 | 2160 | 0.5 | 30–45 |

scheng SDK v0.1.0 — GPU-Accelerated Video Synthesis